ViscaSelect uses various python scripts, VMD (Visual Molecular Dynamics) and the visca Fortran script to compare experimental and calculated VSFG spectra across different polarisation combinations. The package is run through the terminal/command line and, once installed, the command “visca” will be globally available.

The required inputs include:

1) a protein structure file (.pdb)

2) a molecular dynamics trajectory (.dcd)

3) experimental VSFG spectra files (separate single column .txt files for each polarisation and the shared frequency axis- no headers)

4) an input file (ViscaSelect_parameters.inp) containing all the necessary settings explained below.

The main script for ViscaSelect creates a working directory based on the pdb structure name.

The first part of this script calls VMD to split the total trajectory into its consituent frames which are saved as individual .dcd files.

Then the visca fortran script is used to calculate VSFG spectra  for each of these frames.

This produces .txt files for each rotational state containing the lab-frame components of chi(2).

These spectra are also corrected for the local electric fields (by calculating Fresnel factors based on input parameters) and normalised with respect to one of the polarisation combinations (chosen in the the input file).

Residual sum of squares values are calculated by comparing the normalised experimental spectra with the normalised calculated spectra for each frame to quantify the deviation.

Example command: visca select ViscaSelect_parameters.inp

The RSS values can be plotted as a function of simulation time to give a RSS plot using RSS plotter. Note, the time steps and axis labels will need to be adjusted in this python script to give an accurate x-axis for non-example data.

Example command: visca select_RSS_plot ViscaSelect_parameters.inp

Experimental and calculated spectra for a particular frame can be plotted using the [;ptter and specifying the initial and final frame numbers as arguments.

Example command: visca select_plotter ViscaSelect_parameters.inp 0 1

All of the best matching spectra (below a specified RSS cutoff) can be plotted as a shaded region around the best matching spectrum using the shaded plotter.

Example command: visca select_shaded_plot ViscaSelect_parameters.inp

In order to automatically determine a suitable RSS cutoff from the standard deviation in the experimental spectra the cutoff command can be used. This gives a RSS cutoff value, within which the calculated spectra vary as much as the experimental ones do. This cutoff value will need to manually be entered back into the input file and “select_shaded_plot” command run again.

Example command: visca select_RSS_cutoff ViscaSelect_parameters.inp

Example input file parameters are shown below:

name = 6OR3_open_wt_final_charmm_names #refers to the pdb file name (without the .pdb extension at the end).

pdb_location = /home/user/Desktop/ViscaSelect/MD_frames#is the path of the pdb structure file.

dcd_location = /home/user/Desktop/ViscaSelect/MD_frames#is the path of the MD trajectory .dcd file

steps =   100 #the number of frames you would like to include in the calculations. A value of 100 here means only the first 100 frames will be analysed

FPSplit = 1 #number of adjacent frames to concatenate to average their spectral response

RSS_range_i = 1575

RSS_range_f = 1680 #refers to the frequency range (initial and final values) for calculating the RSS values between the experimental and calculated spectra. For example, this can be useful if there is lipid mode close to the amide mode and you would not like it to be included in the RSS calculation - just set the range to exclude the lipid mode.

exp_data_location = /home/user/Desktop/ViscaSelect/Experimental_data/ #is the path for the experimental data. This should be a directory containing separate, single-column, no-header .txt files for the frequecy axis and each polarisation combination.

xvals = xvals.txt

SSP = SSP.txt

PPP = PPP.txt

SPS = SPS.txt

SSP_bg=0.0 #optional value for background to subtract.

PPP_bg=0.0

SPS_bg=0.0

Normalize_wrt = SSP #polarisation combination to normalise with respect to – normally the most intense one.

fortran_script = visca.out #file name of compiled fortran code.

fortran_script_location = /home/user/Desktop/ViscaSelect/ #path for fortran script.

tcl_scripts_location = /home/user/Desktop/ViscaSelect/ #path for the VMD .tcl scripts.

vmd_command = vmd #terminal command for vmd. This is just “vmd” in Linux operating systems, but a path may need to be specified in MacOS.

post_clean = True #optional command to delete .dcd and calculated spectra files generated during analysis to save storage space. Note that the plotter can recalculate these individually as required so we recommend leaving this as True.

RSS_file = RSS.txt  #file name for output continaing RSS values.

RSS_cutoff = 4 #the RSS cutoff value used for determining a good match to experimental structures. This value influences where the dotted cutoff line is drawn on the RSS plot from "Make_RSS_plot.py" and which spectra will be included in the shaded regions in "Make_shaded_plot.py".

exp_std = 0.000555 # a value for the experimental standard deviation which can be used for an automated method for determining a consistent RSS_cutoff value.

width = 18 #Lorentzian linewitdh (cm-1).

mOH = 400 #measure of hydrogen bonding effect (frequency shift per Å).

Omega0 = 1665 #local mode central frequency (cm-1).

lambda_vis = 800 #visible wavelength (nm)

omega_ir = 1665 #ir frequency (cm-1)

n1_sfg = 1 #refractive index of medium 1 at sfg wavelength (here air)

n1_vis = 1 #refractive index of medium 1 at vis wavelength (here air)

n1_ir = 1 #refractive index of medium 1 at ir wavelength (here air)

n2_sfg = 1.3308 #refractive index of medium 2 at sfg wavelength (here water)

n2_vis = 1.329 #refractive index of medium 2 at vis wavelength (here water)

n2_ir = 1.265 #refractive index of medium 2 at ir wavelength (here water)

ni_sfg = 1.47 #interfacial refractive index at sfg wavelength (here fixed based on Voros 2004 Biophys. J. 87 p553-561)

ni_vis = 1.47 #interfacial refractive index at vis wavelength

ni_ir = 1.47 #interfacial refractive index at ir wavelength

theta1_vis = 39 #incidence angle for visible relative to surface normal in degrees.

theta1_ir = 27 #incidence angle for ir relative to surface normal in degrees.